# Goals

HAZARD: Earthquake

- What are the available parameters for an earthquake?

ASSET: Bridges

- What are the available parameters for a bridge?

FRAGILITY: Bridges exposed to earthquakes

- What parameters of an earthquake can induce failure of a bridge system?

# Hazard: Earthquakes

What are the available parameters for an earthquake?

### 1) Event metadata

Data: origin time, Mw, epicenter (lat/lon), depth, event IDs
Sources:

- USGS event page (official event parameters). (earthquake.usgs.gov)

### 2) Shaking intensity measures (primary model input)

Data: ShakeMap grids for PGA and (if available) Sa(T) (e.g., Sa(0.3s), Sa(1.0s)); optional PGV/MMI
Sources:

- USGS Northridge ShakeMap downloads. (earthquake.usgs.gov)

### 3) Strong-motion station records

Data: station locations + recorded PGA / response spectra (processed products preferred)
Sources:

- COSMOS Strong Motion Center (event-based station products). (strongmotioncenter.org)

- USGS NSMP Northridge strong-motion dataset page. (escweb.wr.usgs.gov)

- California Geological Survey processed strong-motion report pages (context + processing). (conservation.ca.gov)

### 4) Site condition proxy at bridge locations

Data: Vs30 at each bridge point
Sources:

- USGS California Vs30 dataset release. (usgs.gov)

- California state GIS site-condition layers (optional). (gis.data.ca.gov)

- SCEC UCVM (optional advanced site model queries). (southern.scec.org)

# Asset: Bridges

# Relevant Bridge Parameters to Explore Further

- Pier Height

  - NBI provides vertical clearance, we could make an assumption that this is a close approximation for pier height

- Pier Capacity

  - I think this will be too complicated to find data for a model

- Bearing Type

  - This is important, hopefully we can find data for this

- Bearing Capacity

  - I think this will be too complicated to find data for a model

- Span Length

  - NBI should provide this

- Skew

  - NBI should provide this

- Span Type (simple or continuous)

  - Hopefully we can find data for this

- Soil Type

  - Hopefully we can find CA soil data for regions

  - Is this too complicated for our model?

- Distance to Epicenter

  - NBI should provide coordinates of bridges

# What are the primary failure mechanisms for bridges subject to earthquake loading?

- Pier/Column Failure (1)

  - Caused by strong lateral ground accelerations, producing large shear and bending moments

  - Key parameters: pier height, cross sectional geometry, material strengths, foundation details

- Superstructure Unseating and Deck Loss (2) (3)

  - Superstructure gets pushed off of supports

  - Caused by excessive displacement exceeds support length, or bearing capacity

    - Most relevant on simply supported bridges

  - Key parameters: length of bearings/supports, bearing stiffness and capacity, span length and geometry

- Bearing and Shear Key Failures (4)

  - Bearing failure or detachment

  - Key parameters: bearing type

- Liquefaction and Ground Failure (5)

  - Seismic shaking can cause soil liquefaction

  - Key parameters: soil type, groundwater depth, embedment depth

- Key Structural Parameters

  - Span Length

  - Superstructure Configuration (simply-supported or continuous)

    - Simply-supported structures have a higher risk of unseating from displacements

  - Pier Height

    - Taller piers experience greater displacements at the top of the substructure

  - Skew Angle (6)

    - Skewed bridges have decreased seismic capacity

  - Bearing Properties

    - Type, stiffness, capacity

  - Support Length

  - Concrete and Steel capacities

  - Soil properties

  - Foundation depth

- Primary Intensity Measures

  - Peak ground velocity, spectral acceleration

- Summary

  - Pier height & stiffness (ductility capacity)

  - Bearing type & support length

  - Span length & geometry (skew)

  - Soil type & liquefaction potential

  - Distance to fault & vertical motion content

What are the available parameters for a bridge?

Preliminary List (not verified)

- Superstructure

  - Number of spans

  - Span length

  - Support type

    - Simply supported, continuous

  - Material strengths

    - Concrete

    - Steel

  - Girder type

    - Concrete

    - Steel

    - Other

  - Number of girders

  - Span configuration

    - Tangent

    - Curved

- Substructure

  - Pier type

    - Wall

    - Column

    - Box girder

  - Pier height

  - Pier thickness

  - Support type

    - Footing

    - Deep foundation

    - Piles

    - Micropiles

  - Material strengths

- Other

  - Geographic location

# Possible data sources

FHWA National Bridge Inventory:

https://www.fhwa.dot.gov/bridge/nbi/disclaim.cfm?nbiYear=1994&nbiState=CA94

- Assuming we want data from the year of the earthquake, 1994?

CALTRANS Bridge Inventory:

https://data.ca.gov/dataset/state-highway-bridges/resource/7fb1bebe-a54c-44f3-ac5e-1bb4636842ed

# NBI: CA25

Parameters from NBI data on California highway bridges in 2025

https://www.fhwa.dot.gov/bridge/nbi/disclaim.cfm?nbiYear=2025/delimited&nbiState=CA25

N/A

UNSURE

RELEVANT

STATE_CODE_001,

STRUCTURE_NUMBER_008,

RECORD_TYPE_005A,

ROUTE_PREFIX_005B,

SERVICE_LEVEL_005C,

ROUTE_NUMBER_005D,

DIRECTION_005E,

HIGHWAY_DISTRICT_002,

COUNTY_CODE_003,

PLACE_CODE_004,

FEATURES_DESC_006A,

CRITICAL_FACILITY_006B,

FACILITY_CARRIED_007,

LOCATION_009,

MIN_VERT_CLR_010,

KILOPOINT_011,

BASE_HWY_NETWORK_012,

LRS_INV_ROUTE_013A,

SUBROUTE_NO_013B,

LAT_016,LONG_017,

DETOUR_KILOS_019,

TOLL_020,

MAINTENANCE_021,

OWNER_022,

FUNCTIONAL_CLASS_026,

YEAR_BUILT_027,

TRAFFIC_LANES_ON_028A,

TRAFFIC_LANES_UND_028B,

ADT_029,

YEAR_ADT_030,

DESIGN_LOAD_031,

APPR_WIDTH_MT_032,

MEDIAN_CODE_033,

DEGREES_SKEW_034,

STRUCTURE_FLARED_035,

RAILINGS_036A,

TRANSITIONS_036B,

APPR_RAIL_036C,

APPR_RAIL_END_036D,

HISTORY_037,

NAVIGATION_038,

NAV_VERT_CLR_MT_039,

NAV_HORR_CLR_MT_040,

OPEN_CLOSED_POSTED_041,

SERVICE_ON_042A,

SERVICE_UND_042B,

STRUCTURE_KIND_043A,

STRUCTURE_TYPE_043B,

APPR_KIND_044A,

APPR_TYPE_044B,

MAIN_UNIT_SPANS_045,

APPR_SPANS_046,

HORR_CLR_MT_047,

MAX_SPAN_LEN_MT_048,

STRUCTURE_LEN_MT_049,

LEFT_CURB_MT_050A,

RIGHT_CURB_MT_050B,

ROADWAY_WIDTH_MT_051,

DECK_WIDTH_MT_052,

VERT_CLR_OVER_MT_053,

VERT_CLR_UND_REF_054A,

VERT_CLR_UND_054B,

LAT_UND_REF_055A,

LAT_UND_MT_055B,

LEFT_LAT_UND_MT_056,

DECK_COND_058,

SUPERSTRUCTURE_COND_059,

SUBSTRUCTURE_COND_060,

CHANNEL_COND_061,

CULVERT_COND_062,

OPR_RATING_METH_063,

OPERATING_RATING_064,

INV_RATING_METH_065,

INVENTORY_RATING_066,

STRUCTURAL_EVAL_067,

DECK_GEOMETRY_EVAL_068,

UNDCLRENCE_EVAL_069,

POSTING_EVAL_070,

WATERWAY_EVAL_071,

APPR_ROAD_EVAL_072,

WORK_PROPOSED_075A,

WORK_DONE_BY_075B,

IMP_LEN_MT_076,

DATE_OF_INSPECT_090,

INSPECT_FREQ_MONTHS_091,

FRACTURE_092A,

UNDWATER_LOOK_SEE_092B,

SPEC_INSPECT_092C,

FRACTURE_LAST_DATE_093A,

UNDWATER_LAST_DATE_093B,

SPEC_LAST_DATE_093C,

BRIDGE_IMP_COST_094,

ROADWAY_IMP_COST_095,

TOTAL_IMP_COST_096,

YEAR_OF_IMP_097,

OTHER_STATE_CODE_098A,

OTHER_STATE_PCNT_098B,

OTHR_STATE_STRUC_NO_099,

STRAHNET_HIGHWAY_100,

PARALLEL_STRUCTURE_101,

TRAFFIC_DIRECTION_102,

TEMP_STRUCTURE_103,

HIGHWAY_SYSTEM_104,

FEDERAL_LANDS_105,

YEAR_RECONSTRUCTED_106,

DECK_STRUCTURE_TYPE_107,

SURFACE_TYPE_108A,

MEMBRANE_TYPE_108B,

DECK_PROTECTION_108C,

PERCENT_ADT_TRUCK_109,

NATIONAL_NETWORK_110,

PIER_PROTECTION_111,

BRIDGE_LEN_IND_112,

SCOUR_CRITICAL_113,

FUTURE_ADT_114,

YEAR_OF_FUTURE_ADT_115,

MIN_NAV_CLR_MT_116,

FED_AGENCY,

SUBMITTED_BY,

BRIDGE_CONDITION,

LOWEST_RATING,

DECK_AREA

# CALTRANS

X

Y

OBJECTID

DIST

CO

PM

BRIDGE

CITY

LAT

LON

NAME

LOC

YRBLT

FAC

LENG

MAINSPANS

APPSPANS

DECKWIDTH

DESIGN_MAIN

MATERIAL_MAIN

INTERSEC

DATA_EXTRACTED

RTE

# Fragility

What parameters of an earthquake can induce failure of a bridge system?

# Some links

### 1. National Bridge Inventory (NBI) – Bridge Inventory Data

- Download annual ASCII data from FHWA (full inventory)
Download NBI ASCII Files (FHWA)
(Includes multiple years of bridge data with attributes used for structural/damage analysis)

- National Bridge Inventory metadata and dataset overview (Data Catalog)
NBI Dataset – data.gov catalog page
(Provides general description, metadata, and links to downloads)

- Optional extra bridge element-level data (condition components)
NBI Element Data Downloads (FHWA)
(Detailed element attributes like deck, superstructure, etc.)

### 2. HAZUS Fragility & Inventory Reference Data

- Hazus-MH download and information (FEMA)
HAZUS Program and Data (FEMA)
(Includes links to Hazus software and inventory database for multi-hazard risk evaluation)

- Hazus Inventory Technical Manual (PDF)
Hazus 6.1 Inventory Technical Manual (PDF)
(Documentation explaining dataset structure used in HAZUS models)

### 3. Hazard Data (Earthquake, Flood, Weather)

Earthquake Data

- USGS Earthquake Hazards Data & Tools (download hazard datasets)
USGS Earthquake Hazards Data Page
(Access real-time and historical seismic hazard data, catalogs, and maps)

Flood Hazard Data

- FEMA National Flood Hazard Layer (NFHL)
National Flood Hazard Layer (GIS Data)
(Flood zone maps and shapefiles for flood risk analysis)

- FEMA Flood Maps viewer & shapefile downloads
FEMA NFHL Viewer & Download Page
(Browse/download geospatial flood hazard data)

Real-Time Water / Flood Conditions (useful for temporal analysis)

- National Water Prediction Service – NOAA
NOAA National Water Prediction Service
(Flood inundation mapping & water level forecast tools)

### Optional Spatial & GIS Resources

Although not direct downloadable datasets, these are useful for GIS compatibility:

- U.S. National Bridge Inventory visual explorer (ArcGIS)
NBI Interactive Map (ArcGIS Experience)
(Visual exploration of bridges; can be useful as a base map)